SOP2 Version 1.1 dated 28-06-05

- Laboratory Rules (Containment Level 1)

2nd Floor Laboratory ERC

University of Manchester

In accordance with “The management, design and operation of microbiological containment laboratories. Advisory Committee on Dangerous Pathogens (ACDP). 2001. HSE Books. ISBN 0 7176 2034 4.” and “Advisory Committee on Genetic Modification (ACGM) Compendium of Guidance. 2000. HSE Books. ISBN 0-7176-1763-7.”

Author: Dr P Bowyer

Next review 2012

Notes: Reviewed 16/1/06

reviewed Jan 2008

reviewed Jan 2010 no changes

1. 		Eating, chewing, drinking, smoking, applying cosmetics, storing of food and outdoor clothing in the laboratory is banned.

2.		Mouth pipetting must not be used under any circumstances for any reason.

3.  		All workers in the laboratory must cover cuts and abrasions with a waterproof dressing.

4.		Wash hands regularly and always before leaving the laboratory.

5.		Laboratory coats must be worn at all times whilst in the laboratory and removed before leaving.

6.		Sharps should not be used unless there is no alternative.  If sharps are used then they must be placed directly in sharps bins for disposal.

7.		On completion of work the work station and all equipment must be cleaned.

8. 		All waste materials must be made safe before disposal.

9.		All specimen containers, glassware and used equipment must be immersed in disinfectant 1% Virkon for at least 2 hours before cleaning or disposal.

10.		Surfaces must be disinfected with 1% Virkon following spillages.

11.		Accidents:

i)	In the event of an accident resulting in a wound immediately encourage it to bleed, wash thoroughly with soap and water but DO NOT SCRUB, cover with waterproof dressing.

ii)	In the event of contamination of skin, conjunctivae or mucous membrane immediately wash thoroughly.

iii)  	Accidents must be reported to ...........................................................

- The above rules must be followed whilst in the laboratory.

............................................

Head of Laboratory